CSC 828 ASSIGNMENTS

BY

BADMUS OLALEKAN SAHEED

COMPUTER SCIENCE DEPARTMENT

COLLEGE OF POSTGRADUATE STUDIES

REDEEMER’S UNIVERSITY

EDE, OSUN STATE, NIGERIA

LECTURER NAME

DR. S.A. ADEPOJU

sa.adepoju@yahoo.co.uk

QUESTIONS

1. What is TCP?

2. What is UDP?

3. Discuss the difference between Connection oriented and connectionless protocols and give examples.

4. Provide a diagram of TCP/IP.

5. Compare and contrast layers both in TCP and OSI models (What make up each layers in OSI and vice versa).

6. Compare IPV4 and IPV6.

ANSWERS

## 1. What is TCP? TCP (Transmission Control Protocol) is a core transport - layer protocol in the Internet protocol suite. It is connection oriented, meaning a connection must be established before data transfer is possible. TCP ensures:

- Reliable delivery (lost packets are retransmitted)

- Ordered delivery (packets arrive in the order sent)

- Error checking (checksums, sequence numbers)

- Flow control (to prevent sender overwhelming the receiver)

- Congestion control (to avoid network overload)

## 2. What is UDP?

UDP (User Datagram Protocol) is a simpler, connectionless transport protocol. Its main characteristics include:

- No connection setup: each datagram is sent independently.

- No guarantee of delivery, ordering, or error recovery (beyond a checksum).

- Minimal overhead (no retransmissions, no flow control).

- Useful when speed is critical and occasional packet loss is acceptable.

## 3. Differences between Connection - Oriented and Connectionless Protocols

| Characteristic | Connection - Oriented Protocols | Connectionless Protocols |
| --- | --- | --- |
| Setup / Teardown | Requires explicit connection setup and teardown | No setup or teardown phase |
| Reliability / Error Control | Retransmissions, acknowledgments, error detection | Basic error detection (e.g., checksum) only |
| Ordering | Guarantees in-order delivery | Packets may arrive out of order |
| Overhead / Complexity | Higher due to state, control messages | Lower, simpler design |
| State Maintenance | Endpoints keep state (connection info) | Stateless per packet |
| Examples | TCP, SCTP | UDP, raw IP (datagram service) |

## 4. Diagram of the TCP/IP Model

Below is a textual diagram of the 4-layer TCP/IP model:

+---------------------+----------------------+--------------------+-----------------------------+

| Application Layer   | Transport Layer      | Internet Layer        | Network Access Layer        |

+---------------------+----------------------+--------------------+-----------------------------+

| HTTP, FTP, DNS, SMTP| TCP, UDP             | IP (IPv4 / IPv6), ICMP, IGMP | Ethernet, Wi-Fi, ARP, PPP |

+---------------------+----------------------+--------------------+-----------------------------+

## 5. Comparing: TCP/IP Model vs OSI Model

### OSI Model Layers & Functions

- Physical — raw bit transmission over physical medium (cables, signal levels)

- Data Link — framing, MAC addressing, error detection/correction, link control

- Network — routing, addressing, packet forwarding

- Transport — end-to-end communication, reliability, flow control

- Session — session establishment, maintenance, termination

- Presentation — data translation, encryption, compression

- Application — interface for applications, network services

### TCP/IP Model & Mapping

| OSI Layer(s) | TCP/IP Layer | Functions / Protocols Included |
| --- | --- | --- |
| Application, Presentation, Session | Application Layer | HTTP, FTP, DNS, SMTP, TLS/SSL, etc. |
| Transport | Transport Layer | TCP, UDP |
| Network / Internet | Internet Layer | IP (IPv4, IPv6), ICMP, IGMP, routing |
| Data Link + Physical | Network Access Layer | Ethernet, Wi-Fi (802.11), ARP, PPP, framing, encoding |

Key Differences:

- The TCP/IP model merges OSI’s top three layers (Application, Presentation, Session) into one Application layer.

- TCP/IP’s Network Access (or Link) layer encompasses both Physical and Data Link layers of OSI.

- TCP/IP is more pragmatic and implementation driven, whereas OSI is more theoretical and descriptive.

## 6. Comparing IPv4 vs IPv6

| Feature | IPv4 | IPv6 |
| --- | --- | --- |
| Address Size | 32 bits (4 octets) | 128 bits (16 bytes) |
| Address Notation | Dotted decimal (e.g. 192.168.0.1) | Hexadecimal, colon notation (e.g. 2001:0db8::1) |
| Address Space | ~4.3 billion addresses | Immense (2^128 addresses) |
| Header Complexity | Variable header with options, includes header checksum | Fixed header (simpler), no header checksum |
| Broadcast Support | Supports broadcast | No broadcast; uses multicast / anycast |
| Configuration | Manual, DHCP | Stateless address autoconfiguration (SLAAC), DHCPv6 |
| Fragmentation | By sender and routers | Only by sender; routers do not fragment |
| NAT Necessity | Common (due to scarcity of addresses) | Less or no need for NAT |
| IPv6-only Features | — | Built-in IPsec, better support for mobility, flow labels |
| Compatibility Strategy | — | Dual stack, tunneling, translation mechanisms |